HSIP-Secondary Funding Stipulations

For paving projects with a FINAL paved top width less than 26 feet:

- HSIP-S will participate in the following improvements:

  - Pavement/base widening to achieve up to 11-foot lanes

  - Paved shoulder construction and/or widening to achieve up to 2-foot paved shoulders*

  - Safety Edge (regardless of paved shoulder width or presence)

  - Rumble strips (centerline, edgeline, and shoulder)

  - Grooving for pavement markings (centerline, edgeline, and transverse markings)

- Projects shall include:

  - Safety Edge (where paved shoulder width is less than 4 feet)

  - Six-inch edgeline markings (either in a 6-inch groove or in a rumble strip)

  - One or both of the following:

    - Centerline rumble strips (PV-13 or similar design)

    - Shoulder or edgeline rumble strips (PV-12 or similar design, 6-inch minimum width)

For paving projects with a FINAL paved top width 26 feet or greater:

- HSIP-S will participate in the following improvements:

  - Pavement/base widening to achieve up to 12-foot lanes

  - Paved shoulder construction and/or widening to achieve up to 2-foot paved shoulders*

  - Safety Edge (regardless of paved shoulder width or presence)

  - Rumble strips (centerline, edgeline, and shoulder)

  - Grooving for pavement markings (centerline, edgeline, and transverse markings)

- Projects shall include:

  - Safety Edge (where paved shoulder width is less than 4 feet)

  - Six-inch edgeline markings (either in a 6-inch groove or in a rumble strip)

  - Both of the following:

    - Centerline rumble strips (PV-13 or similar design)

    - Shoulder or edgeline rumble strips (PV-12 or similar design, 6-inch minimum width)

For pavement marking projects:

- HSIP-S will participate in the following improvements:

  - Grooving for pavement markings (centerline, edgeline, and transverse markings)

  - Initial application of pavement markings

  - Centerline rumble strips

  - Shoulder or edgeline rumble strips

- Projects shall include:

  - Centerline rumble strips (PV-13 or similar design)

  - Six-inch edgeline markings

  - One of the following:

    - Grooved-in edgeline markings

    - Shoulder or edgeline rumble strips (PV-12 or similar design, 6-inch minimum width)

*Up to 4-foot paved shoulders within horizontal curves